Chemical Engineering 374

Reading Questions 16—Chapter 8.8

Name ________________________________________________________

- How do the 3 common types of obstruction meters—orifice, nozzle and venturi—compare in terms of friction head loss?

- Which type of flowmeters are well suited for tracking the cumulative amount of fluid passing through it (as opposed to the rate of flow)?

- For a vertical rotameter, what opposing forces influence the position of the float?

- In what way do Doppler-effect ultrasonic flowmeters depend on impurities in the flowing fluid in order to make their measurement?

- Why are electromagnetic flowmeters suitable to measure the flow rate of seawater, but not the flow rate of distilled water?

- What parameter is varied to maintain the temperature of a hot-wire anemometer at a constant value?  How does that parameter aid in determining the flow velocity?

- What restrictions apply to the seeding particles used in LDV and PIV?